HOLLISTER INCORPORATED

Event-Driven Architecture

Design Guide

MQTT + Kafka Hybrid Architecture

From Shop Floor to Boardroom

The Architecture Metaphor MQTT is the Nervous System Fast, lightweight, responsive to the "here and now." Kafka is the Brain and Memory Capable of storing, analyzing, and replaying everything that has happened

| Document ID | HOLL-ARCH-EDA-001 |
| --- | --- |
| Version | 1.0 |
| Date | February 2026 |
| Related Documents | HOLL-ARCH-001, HOLL-TECH-PLC-MES-001 |

# Table of Contents

# 1. Executive Overview

## 1.1 The Integration Challenge

Hollister’s Digital Factory transformation requires seamless data flow from the shop floor (PLCs, sensors, MES) to enterprise systems (ERP, CRM, PLM, Supply Chain). This presents a fundamental architectural challenge: shop floor systems operate in milliseconds with high-frequency, ephemeral data, while enterprise systems operate in minutes or hours with durable, transactional data.

A single messaging technology cannot optimally serve both worlds. Attempting to force-fit one solution leads to either:

- Using MQTT for everything: Loses data durability, replay capability, and enterprise integration patterns

- Using Kafka for everything: Adds unnecessary latency and complexity at the edge, overwhelms brokers with millions of tiny topics

## 1.2 The Hybrid Solution

This guide defines a Hybrid Event-Driven Architecture (EDA) that uses the right tool for each layer:

| Layer | Technology | Why |
| --- | --- | --- |
| Edge/Shop Floor | MQTT | Lightweight, handles unreliable networks, millisecond response, millions of topics |
| Integration Bridge | Kafka Connect / Bridge | Aggregates, filters, transforms, and routes data between worlds |
| Enterprise | Apache Kafka | Durable storage, replay capability, exactly-once semantics, enterprise consumers |

# 2. MQTT vs. Kafka: Understanding the Differences

While both MQTT and Kafka use "topics" to categorize data, they serve fundamentally different purposes. Understanding these differences is critical to designing an effective hybrid architecture.

## 2.1 Comprehensive Feature Comparison

| Feature | MQTT | Kafka |
| --- | --- | --- |
| Primary Role | Real-time messaging for IoT/edge devices | Durable event streaming and log storage |
| Topic Structure | Hierarchical: factory/line/machine/sensor. Supports wildcards (+, #) | Flat: machine_events. No hierarchy; partitioning handles scaling |
| Message Persistence | Ephemeral: Gone after delivery (unless Retained flag). QoS ensures delivery, not storage | Durable: Stored on disk for configurable retention (days, months, years). Full replay capability |
| Delivery Model | Push: Broker immediately pushes to active subscribers | Pull: Consumers request data when ready, at their own pace |
| Topic Scaling | Scales to millions of topics easily. Each sensor can have its own topic | Optimized for high throughput per topic. Limited total topic count (thousands) |
| Message Size | Optimized for small messages (bytes to KB) | Handles larger messages efficiently (KB to MB) |
| Network Tolerance | Excellent: Designed for unreliable, high-latency networks. Small protocol overhead | Good: Requires stable network. Higher protocol overhead |
| Consumer Groups | Limited: Shared subscriptions (MQTT 5.0) provide basic load balancing | Advanced: Consumer groups with partition assignment, rebalancing, offset management |
| Ordering Guarantees | Per-topic ordering only | Per-partition ordering with configurable partitioning strategy |
| Replay Capability | None: Messages are fire-and-forget (Retained gives only last message) | Full: Consumers can rewind to any offset and replay history |
| Latency | Sub-millisecond to low milliseconds | Low milliseconds to tens of milliseconds |
| Best For | PLCs, sensors, edge devices, MES real-time, mobile/constrained clients | ERP integration, analytics, audit logs, event sourcing, microservices |

## 2.2 Topic Namespace Isolation

A critical point: MQTT topics and Kafka topics exist in completely separate namespaces. They are not shared natively and must be explicitly bridged.

This means:

- An MQTT topic factory1/line3/plc/temp has no relationship to a Kafka topic of the same name

- Data must flow through a bridge component that subscribes to MQTT and publishes to Kafka

- Topic mapping and transformation logic lives in the bridge layer

# 3. The Hybrid Architecture

## 3.1 Three-Layer Design

The hybrid architecture consists of three distinct layers, each optimized for its specific requirements:

┌───────────────────────────────────────────────────────────────────────────┐ │ ENTERPRISE LAYER (Kafka) │ │ ┌────────────┐ ┌────────────┐ ┌────────────┐ ┌────────────┐ ┌─────────┐ │ │ │ SAP ERP │ │ CRM │ │ PLM │ │Supply Chain│ │Analytics│ │ │ │ S/4HANA │ │ Salesforce │ │ Teamctr │ │ SCM │ │ AI/ML │ │ │ └──────┬─────┘ └──────┬─────┘ └──────┬─────┘ └──────┬─────┘ └────┬────┘ │ │ │ │ │ │ │ │ │ └────────────┴──────┬─────┴────────────┴───────────┘ │ │ │ PULL │ │ ┌───────────────────────────┴───────────────────────────────────┐ │ │ │ APACHE KAFKA CLUSTER │ │ │ │ Topics: production_events, quality_events, equipment_state, │ │ │ │ inventory_transactions, maintenance_events │ │ │ │ Retention: 30 days (configurable) ¦ Replication: 3 │ │ │ └───────────────────────────────┬───────────────────────────────┘ │ └─────────────────────────────────┼─────────────────────────────────────────┘ │ ┌─────────────────────────────────┼─────────────────────────────────────────┐ │ INTEGRATION BRIDGE │ │ ┌───────────────────────────────────────────────────────────────────┐ │ │ │ Kafka Connect / MQTT-Kafka Bridge / Custom Stream Processor │ │ │ │ │ │ │ │ Functions: │ │ │ │ • Subscribe to MQTT topics (wildcard: factory/#) │ │ │ │ • Aggregate granular MQTT topics → broader Kafka topics │ │ │ │ • Filter noise (deadband, sampling) │ │ │ │ • Transform/enrich with context │ │ │ │ • Schema validation and conversion │ │ │ └───────────────────────────────┬───────────────────────────────────┘ │ └─────────────────────────────────┼─────────────────────────────────────────┘ │ SUBSCRIBE ┌─────────────────────────────────┼─────────────────────────────────────────┐ │ EDGE LAYER (MQTT) │ │ ┌───────────────────────────────────────────────────────────────────┐ │ │ │ MQTT BROKER (HiveMQ / EMQX) │ │ │ │ Topics: factory/+/line/+/machine/+/# (hierarchical, granular) │ │ │ └─────────────┬───────────────────┬────────────────┬───────────────┘ │ │ │ │ │ PUSH │ │ ┌─────────────┴───┐ ┌───────────┴──────┐ ┌──────┬───────────────┐ │ │ │ Critical Mfg MES │ │ Real-Time Dashboard│ │Digital│ Alarm Mgmt │ │ │ │ (Real-time ops) │ │ (Operator screens) │ │ Twin │ (Immediate)│ │ │ └──────────────────┘ └───────────────────┘ └──────┴───────────────┘ │ │ │ │ PUBLISH PUBLISH PUBLISH │ │ ┌─────────────┐ ┌─────────────┐ ┌─────────────┐ ┌─────────────┐ │ │ │ PLC 1 │ │ PLC 2 │ │ Sensors │ │ Vision │ │ │ │Allen-Bradley│ │ Siemens │ │ (Temp/Press)│ │ Systems │ │ │ └─────────────┘ └─────────────┘ └─────────────┘ └─────────────┘ │ └───────────────────────────────────────────────────────────────────────────┘

# 4. Edge Layer: MQTT for Shop Floor

## 4.1 Why MQTT at the Edge

PLCs (like Siemens S7, Allen-Bradley ControlLogix) and shop floor sensors operate in resource-constrained, often unreliable network environments. MQTT is purpose-built for this world:

- Lightweight Protocol: Minimal overhead (2-byte header minimum), perfect for constrained devices

- Unreliable Networks: Built-in QoS levels handle packet loss, connection drops gracefully

- Millisecond Response: MES can react to machine state changes in real-time

- Hierarchical Topics: Natural mapping to physical plant structure (site/area/line/machine/sensor)

- Wildcard Subscriptions: MES subscribes to factory/+/+/state to monitor all machines

## 4.2 MQTT Topic Design for Manufacturing

The MQTT topic hierarchy mirrors the physical plant structure and ISA-95 equipment model:

hollister/ ├── {site}/ # BAL, SDR, LIB, KAU, FRE │ ├── {area}/ # MH1 (Molding Hall 1), PKG, CLN │ │ ├── {line}/ # LINE01, LINE02 │ │ │ ├── {equipment}/ # MOLD01, PRESS02, ROBOT03 │ │ │ │ ├── state # Equipment state (PackML) │ │ │ │ ├── telemetry/ # Real-time sensor data │ │ │ │ │ ├── temperature │ │ │ │ │ ├── pressure │ │ │ │ │ ├── vibration │ │ │ │ │ └── speed │ │ │ │ ├── production/ # Production events │ │ │ │ │ ├── cycle_complete │ │ │ │ │ ├── count │ │ │ │ │ └── reject │ │ │ │ ├── quality/ # Quality measurements │ │ │ │ │ ├── inspection │ │ │ │ │ └── spc │ │ │ │ ├── alarms/ # Active alarms │ │ │ │ │ ├── active │ │ │ │ │ └── acknowledged │ │ │ │ └── maintenance/ # Maintenance events │ │ │ │ ├── request │ │ │ │ └── complete Example Subscriptions: • MES subscribes to: hollister/BAL/+/+/+/state (all equipment states in Ballina) • Digital Twin: hollister/BAL/MH1/# (everything in Molding Hall 1) • Alarm System: hollister/+/+/+/+/alarms/active (all active alarms globally)

## 4.3 MQTT QoS Levels for Manufacturing

| QoS | Guarantee | Use Case | Example |
| --- | --- | --- | --- |
| 0 | At most once (fire and forget) | High-frequency telemetry where occasional loss is acceptable | Temperature readings every 100ms |
| 1 | At least once (may duplicate) | Production events where loss is unacceptable | Cycle complete, part count |
| 2 | Exactly once (guaranteed single delivery) | Critical events requiring audit trail | Quality failures, safety alarms |

## 4.4 Edge Consumers (MQTT Subscribers)

These systems subscribe directly to MQTT for real-time responsiveness:

| Consumer | Subscription Pattern | Latency Requirement |
| --- | --- | --- |
| MES (Critical Mfg) | hollister/{site}/+/+/+/production/#, .../state | < 100ms for state changes, cycle complete |
| Digital Twin | hollister/{site}/# | < 500ms for visualization sync |
| Alarm Management | hollister/+/+/+/+/alarms/# | < 50ms for safety-critical alarms |
| Operator Dashboards | hollister/{site}/{area}/# | < 1s for screen updates |
| Edge Analytics | hollister/{site}/+/+/+/telemetry/# | Real-time for SPC, OEE calculation |

# 5. Integration Bridge: Connecting Worlds

## 5.1 Bridge Purpose and Functions

The Integration Bridge is the critical translation layer between the MQTT edge world and the Kafka enterprise world. It performs several essential functions:

| Function | Description |
| --- | --- |
| Topic Aggregation | Consolidate many granular MQTT topics into fewer broad Kafka topics. Example: 1000 machine state topics → 1 Kafka equipment_state topic |
| Filtering | Apply deadband filtering, sampling, and noise reduction. Not every sensor reading needs to reach Kafka—only significant changes |
| Transformation | Convert MQTT JSON payloads to Avro schemas, enrich with metadata, normalize timestamps to UTC |
| Batching | Collect multiple MQTT messages into batches for efficient Kafka writes, reducing broker load |
| Schema Validation | Validate incoming data against registered schemas, reject malformed messages with logging |
| Partitioning | Determine Kafka partition key (typically equipment ID) to ensure ordering guarantees |

## 5.2 Topic Mapping Strategy

The bridge maps the hierarchical, granular MQTT namespace to a flat, coarse Kafka namespace:

| MQTT Topics (Source) | Kafka Topic (Target) |
| --- | --- |
| hollister/+/+/+/+/state | equipment_state |
| hollister/+/+/+/+/production/cycle_complete | production_events |
| hollister/+/+/+/+/production/count, .../reject | production_events |
| hollister/+/+/+/+/quality/# | quality_events |
| hollister/+/+/+/+/alarms/# | alarm_events |
| hollister/+/+/+/+/telemetry/# (filtered) | sensor_readings (sampled) |
| hollister/+/+/+/+/maintenance/# | maintenance_events |

## 5.3 Bridge Implementation Options

| Option | Strengths | Best For |
| --- | --- | --- |
| Kafka Connect MQTT Source | Managed by Kafka ecosystem, declarative config, scales with Kafka | Standard integrations, minimal transformation |
| Confluent MQTT Proxy | Native MQTT interface to Kafka, no separate MQTT broker needed | Simpler architecture, Confluent customers |
| HiveMQ Kafka Extension | Integrated with HiveMQ broker, bidirectional support | HiveMQ deployments, bidirectional flows |
| Custom Stream Processor | Full control over transformation logic, complex enrichment | Advanced filtering, semantic enrichment, custom logic |

## 5.4 Bridge Configuration Example

Example Kafka Connect MQTT Source configuration:

{ "name": "mqtt-kafka-bridge-production", "config": { "connector.class": "io.confluent.connect.mqtt.MqttSourceConnector", "tasks.max": "4", "mqtt.server.uri": "ssl://mqtt-broker.hollister.com:8883", "mqtt.topics": "hollister/+/+/+/+/production/#", "mqtt.qos": "1", "kafka.topic": "production_events", "transforms": "extractFields,addTimestamp,setKey", "transforms.extractFields.type": "org.apache.kafka.connect.transforms.ExtractField$Value", "transforms.extractFields.field": "payload", "transforms.addTimestamp.type": "org.apache.kafka.connect.transforms.InsertField$Value", "transforms.addTimestamp.timestamp.field": "kafka_timestamp", "transforms.setKey.type": "org.apache.kafka.connect.transforms.ValueToKey", "transforms.setKey.fields": "equipment_id", "value.converter": "io.confluent.connect.avro.AvroConverter", "value.converter.schema.registry.url": "https://schema-registry.hollister.com:8081", "key.converter": "org.apache.kafka.connect.storage.StringConverter" } }

# 6. Enterprise Layer: Kafka for Business Systems

## 6.1 Why Kafka for Enterprise

Enterprise systems—ERP, CRM, PLM, Supply Chain—care about the state and history of data rather than individual sensor blips. They need:

- Durability: Events must persist for days/months for audit, replay, and recovery

- Exactly-Once Semantics: Financial transactions cannot be duplicated or lost

- Consumer Independence: Each system consumes at its own pace without affecting others

- Replay Capability: Reprocess historical data when business logic changes

- Integration Patterns: Event sourcing, CQRS, saga patterns for distributed transactions

## 6.2 Kafka Topics for Manufacturing

| Topic | Partitions | Retention | Consumers |
| --- | --- | --- | --- |
| equipment_state | By site (5) | 7 days | ERP (availability), Analytics (OEE) |
| production_events | By equipment (50) | 90 days | ERP (inventory), CRM (order status), PLM (actuals vs design) |
| quality_events | By product (20) | 2 years | QMS, Regulatory (DHR), Analytics (predictive quality) |
| inventory_transactions | By warehouse (10) | 1 year | ERP (stock levels), SCM (replenishment) |
| maintenance_events | By site (5) | 1 year | CMMS, Analytics (predictive maintenance) |
| alarm_events | By severity (3) | 90 days | Analytics (pattern detection), Compliance (safety logs) |

## 6.3 Enterprise Consumer Patterns

### 6.3.1 SAP ERP Integration

When a "Job Finished" event arrives from MES via Kafka:

- Inventory Update: Consume from production_events, update material stock levels in SAP MM

- Cost Allocation: Post labor and machine hours to cost centers

- Production Confirmation: Update production order status, post goods receipt

// SAP Kafka Consumer - Production Event Handler consumer.subscribe(["production_events"]); for await (const message of consumer) { const event = ProductionEvent.deserialize(message.value); if (event.eventType === "BATCH_COMPLETE") { // Post goods receipt to SAP await sapClient.postGoodsReceipt({ material: event.materialNumber, plant: event.site, quantity: event.quantityProduced, batch: event.batchNumber, storageLocation: event.destinationBin }); // Confirm production order await sapClient.confirmProductionOrder({ orderNumber: event.workOrderId, confirmationQuantity: event.quantityProduced, scrapQuantity: event.quantityRejected, activityType: "PROD", laborHours: event.laborMinutes / 60 }); } await consumer.commitOffsets(); }

### 6.3.2 CRM Integration (Order Status)

Customer-facing order status updates flow from production events:

- Order Tracking: When batch completes, update customer order status

- Ship Notification: Trigger shipment notification to customer

- Patient Portal: Update Secure Start™ portal with order progress

### 6.3.3 PLM Integration (Design vs. Actuals)

Product Lifecycle Management uses production data to improve designs:

- Process Capability: Compare actual process parameters to design specifications

- Quality Feedback: Feed defect data back to design for root cause analysis

- Continuous Improvement: Track yield trends against design revisions

### 6.3.4 Supply Chain Integration

Supply chain systems use production and inventory events for planning:

- Demand Signal: Production consumption triggers supplier replenishment

- Capacity Planning: Equipment availability feeds supply/demand balancing

- Supplier Scorecards: Material quality events update supplier performance metrics

# 7. End-to-End Data Flow

## 7.1 Complete Flow Example

This example traces a temperature reading from sensor to ERP:

| Step | Component | Action |
| --- | --- | --- |
| 1 | PLC/Sensor | Temperature sensor reads 185°C on injection molding barrel |
| 2 | Edge Gateway | OPC-UA server polls PLC, converts to MQTT message |
| 3 | MQTT Broker | Receives publish to hollister/BAL/MH1/LINE01/MOLD01/telemetry/temperature |
| 4a | MES (MQTT Sub) | Immediately receives temperature, updates Digital Twin visualization |
| 4b | Edge Analytics | Calculates rolling average, detects 185°C exceeds 180°C limit, publishes alarm |
| 5 | Integration Bridge | Subscribes to telemetry topic, applies deadband filter (change > 2°C), batches for Kafka |
| 6 | Kafka | Bridge publishes to sensor_readings topic with Avro schema, partitioned by equipment_id |
| 7 | Analytics Platform | Kafka consumer builds time-series for predictive model, detects anomaly pattern |
| 8 | Maintenance System | Receives predictive maintenance recommendation, creates work order in SAP PM |

## 7.2 Production Cycle Complete Flow

This example traces a production cycle from completion to ERP booking:

Timeline of a Production Cycle Complete Event: T+0ms PLC detects cycle complete, increments part counter T+5ms Edge gateway reads counter via OPC-UA T+10ms MQTT publish: hollister/BAL/MH1/LINE01/MOLD01/production/cycle_complete Payload: {"count": 1, "cycle_time": 12.3, "material_lot": "LOT-2026-001"} T+15ms MES receives MQTT message (QoS 1) → Updates work order progress → Records cycle time in eDHR → Decrements material lot quantity T+20ms Digital Twin receives update → Increments visual counter → Updates cycle time gauge T+100ms Bridge receives MQTT message → Enriches with work order context from MES API → Transforms to Avro ProductionEvent schema → Publishes to Kafka production_events topic T+150ms Kafka persists message (replication factor 3) → Available for all consumers T+500ms ERP Kafka consumer processes message → Posts goods receipt to SAP MM → Updates inventory levels → Posts production confirmation T+1000ms CRM Kafka consumer processes message → Updates customer order status → Calculates estimated ship date T+2000ms Analytics consumer processes message → Updates OEE calculation → Feeds predictive quality model

# 8. Implementation Guidance

## 8.1 Deployment Architecture

| Component | Deployment Location | Sizing Guidance |
| --- | --- | --- |
| MQTT Broker | Per site (edge data center) | HiveMQ cluster (3 nodes), 100K concurrent connections |
| Edge Gateway | Per plant area | Kepware server with 50K tags per instance |
| Integration Bridge | Per site or centralized | Kafka Connect cluster (3 workers), 4 connectors |
| Kafka Cluster | Centralized (cloud or on-prem) | Confluent Cloud or 5-node cluster, 100K events/sec |
| Schema Registry | With Kafka cluster | 3-node cluster for HA |

## 8.2 Recommended Technology Stack

| Layer | Recommended | Alternatives |
| --- | --- | --- |
| MQTT Broker | HiveMQ Enterprise | EMQX Enterprise, Mosquitto (small scale), AWS IoT Core |
| Edge Gateway | Kepware KEPServerEX | Ignition Edge, Azure IoT Edge, EdgeX Foundry |
| Integration Bridge | Kafka Connect + MQTT Connector | HiveMQ Kafka Extension, Custom Flink/Spark Streaming |
| Event Streaming | Confluent Platform / Apache Kafka | Amazon MSK, Azure Event Hubs (Kafka API), Redpanda |
| Schema Management | Confluent Schema Registry | AWS Glue Schema Registry, Apicurio Registry |

## 8.3 Migration Strategy

For brownfield deployments, follow this phased approach:

Phase 1: Shadow Mode (4 weeks)

- Deploy MQTT broker alongside existing systems

- Configure edge gateways to publish to MQTT (dual-write)

- Validate data flow without impacting production systems

Phase 2: Bridge Activation (4 weeks)

- Deploy Kafka cluster and integration bridge

- Enable bridge to consume MQTT and produce to Kafka

- Connect analytics consumers to Kafka (read-only)

Phase 3: Enterprise Integration (8 weeks)

- Connect ERP/CRM Kafka consumers

- Migrate from legacy point-to-point integrations

- Decommission legacy data paths

Phase 4: Edge Optimization (Ongoing)

- Tune MQTT topic structure based on usage patterns

- Optimize bridge filtering and batching

- Add edge analytics capabilities

# 9. Best Practices and Anti-Patterns

## 9.1 Best Practices

| Practice | Rationale |
| --- | --- |
| Use MQTT for edge, Kafka for enterprise | Each protocol optimized for its domain; don’t force-fit |
| Filter at the edge | Reduce bandwidth and Kafka load by filtering noise before the bridge |
| Aggregate MQTT topics to Kafka | Kafka performs best with fewer, higher-throughput topics |
| Use Avro schemas in Kafka | Schema evolution, validation, and compact encoding for enterprise |
| Partition by equipment ID | Ensures ordering per equipment, enables parallel processing |
| Implement idempotent consumers | Handle at-least-once delivery; use event IDs to detect duplicates |
| Monitor bridge lag | Bridge backlog indicates capacity issues; alert before data loss |
| Test failover scenarios | Validate MQTT broker failover, Kafka consumer rebalancing, bridge recovery |

## 9.2 Anti-Patterns to Avoid

| Anti-Pattern | Why It’s Problematic |
| --- | --- |
| Kafka at the edge | Kafka clients are heavy, require stable networks, and topic count doesn’t scale to millions |
| MQTT for enterprise integration | No persistence, no replay, limited consumer group support for enterprise patterns |
| 1:1 MQTT-to-Kafka topic mapping | Creates topic explosion in Kafka; 1M MQTT topics should map to ~10 Kafka topics |
| No bridge filtering | Flooding Kafka with every sensor reading wastes storage and overwhelms consumers |
| JSON everywhere | JSON is fine for MQTT; use Avro in Kafka for schema enforcement and efficiency |
| Ignoring consumer lag | Slow consumers can fall behind retention; data is lost silently |
| Synchronous enterprise calls from edge | Edge devices should publish events, not call ERP APIs directly |

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| MQTT | Message Queuing Telemetry Transport – lightweight publish-subscribe messaging protocol designed for IoT and constrained devices |
| Kafka | Apache Kafka – distributed event streaming platform for high-throughput, durable, real-time data pipelines |
| QoS | Quality of Service – MQTT delivery guarantee levels (0=at most once, 1=at least once, 2=exactly once) |
| Topic | Named channel for message categorization (hierarchical in MQTT, flat in Kafka) |
| Partition | Kafka concept – ordered, immutable sequence of messages within a topic, enables parallelism |
| Consumer Group | Kafka concept – set of consumers that cooperatively process partitions, enabling load balancing |
| Offset | Position of a message within a Kafka partition, used for replay and consumer tracking |
| Retained Message | MQTT feature – broker stores last message per topic for new subscribers |
| Bridge | Component that transfers data between MQTT and Kafka, performing transformation and filtering |
| Avro | Apache Avro – compact binary serialization format with schema evolution support, commonly used with Kafka |
| Schema Registry | Service that stores and validates message schemas, ensuring producer-consumer compatibility |
| EDA | Event-Driven Architecture – design pattern where systems communicate via events rather than direct calls |

End of Event-Driven Architecture Guide