HOLLISTER INCORPORATED

PLC-to-MES Integration

Technical Specification

Vendor-Neutral Abstraction Architecture

for IIoT, Edge Computing, and Digital Twin Enablement

| Document ID | HOLL-TECH-PLC-MES-001 |
| --- | --- |
| Version | 1.0 |
| Date | February 2026 |
| Classification | Confidential |
| Status | Draft for Review |

Companion Document to:

Digital Factory Solution Architecture (HOLL-ARCH-001)

# Table of Contents

Table of Contents	2

1. Introduction	3

1.1 Purpose	3

1.2 Scope	3

1.3 Design Principles	3

2. Architecture Overview	4

2.1 Layered Architecture Model	4

2.2 Data Flow Architecture	4

3. Physical Device Layer (Layer 1)	5

3.1 PLC Configuration Standards	5

3.1.1 Supported PLC Platforms	5

3.1.2 Tag Naming Convention	5

3.1.3 Tag Naming Elements	5

3.1.4 Equipment State Model (PackML/ISA-88)	6

4. Edge Computing Layer (Layer 3)	7

4.1 Edge Gateway Architecture	7

4.1.1 Edge Gateway Functions	7

4.1.2 Recommended Edge Platforms	7

4.2 Protocol Standards	7

4.2.1 OPC-UA (Primary Standard)	7

4.2.2 MQTT (Event Messaging)	8

5. Semantic and Context Layer (Layer 4)	9

5.1 Canonical Data Model	9

5.1.1 Equipment Hierarchy (ISA-95)	9

5.1.2 Equipment Definition Schema	9

5.2 Metadata Registry	10

5.2.1 Registry Components	10

5.3 Data Governance	10

5.3.1 Governance Policies	11

6. Integration Platform (Layer 5)	12

6.1 Event Streaming Architecture	12

6.1.1 Kafka Topic Structure	12

6.1.2 Message Schema (Avro)	12

6.2 API Gateway	13

6.2.1 API Endpoints	14

7. MES Integration (Layer 6)	15

7.1 MES Adapter Pattern	15

7.1.1 Adapter Responsibilities	15

7.1.2 Critical Manufacturing MES Integration Points	15

7.1.3 Adapter Implementation Pattern	15

8. Security Architecture	17

8.1 OT/IT Network Segmentation	17

8.1.1 Network Zones	17

8.2 Authentication and Authorization	17

8.3 Regulatory Compliance (21 CFR Part 11)	17

9. Implementation Guide	18

9.1 Equipment Onboarding Checklist	18

9.2 Troubleshooting Guide	18

Appendix A: OPC-UA Information Models	19

Appendix B: Reference Architecture Diagram	20

# 1. Introduction

## 1.1 Purpose

This Technical Specification defines the architecture, standards, and implementation guidelines for integrating Programmable Logic Controllers (PLCs) and manufacturing equipment with the Manufacturing Execution System (MES) across all Hollister manufacturing facilities. The specification establishes a vendor-neutral abstraction layer that enables interoperability, prevents vendor lock-in, and provides the foundation for Industrial Internet of Things (IIoT), edge computing, and Digital Twin capabilities.

## 1.2 Scope

This specification covers:

- PLC configuration standards and tag naming conventions

- Edge computing architecture and protocol translation

- Semantic modeling and canonical data structures

- Event-driven integration platform design

- MES integration patterns and adapters

- Data governance and regulatory compliance

- Security architecture for OT/IT convergence

## 1.3 Design Principles

The architecture is built on the following foundational principles:

| Principle | Description |
| --- | --- |
| Abstraction | No direct coupling between MES and physical devices. All integration flows through abstraction layers that normalize protocols and data models. |
| Standards-Based | Use industry standards (OPC-UA, MQTT, ISA-95, ISA-88) over proprietary protocols wherever possible. |
| Event-Driven | Publish-subscribe architecture decouples data producers from consumers, enabling future extensibility. |
| Semantic Context | All data carries meaning through canonical models, metadata, and governed vocabularies. |
| Edge Intelligence | Process data close to the source for latency, bandwidth, and resilience benefits. |
| Governance First | Audit trails, data lineage, and compliance controls are built in from day one. |

# 2. Architecture Overview

## 2.1 Layered Architecture Model

The PLC-to-MES integration architecture consists of six distinct layers, each with specific responsibilities. Data flows upward from physical devices through abstraction and context layers to consuming applications.

| Layer | Name | Responsibility |
| --- | --- | --- |
| 6 | Application Layer | MES, Digital Twin, Analytics, AI/ML consumers |
| 5 | Integration Platform | Event streaming (Kafka), API gateway, message routing |
| 4 | Semantic/Context Layer | Canonical models, metadata registry, governance engine |
| 3 | Edge Computing Layer | Protocol translation, data normalization, local processing |
| 2 | Communication Layer | OPC-UA servers, MQTT brokers, industrial protocols |
| 1 | Physical Device Layer | PLCs, sensors, actuators, equipment controllers |

## 2.2 Data Flow Architecture

Data flows through the architecture in both directions:

Northbound (Device to Enterprise):

- Equipment state changes, production counts, process measurements

- Quality inspection results, alarm events

- Material consumption and tracking events

Southbound (Enterprise to Device):

- Work order downloads and recipe parameters

- Setpoint changes and production commands

- Label data and serialization assignments

# 3. Physical Device Layer (Layer 1)

## 3.1 PLC Configuration Standards

All PLCs integrated into the Digital Factory architecture must conform to these configuration standards to ensure consistent integration and future portability.

### 3.1.1 Supported PLC Platforms

| Vendor | Platform | Protocol | Status |
| --- | --- | --- | --- |
| Rockwell/Allen-Bradley | ControlLogix, CompactLogix | EtherNet/IP, CIP | Preferred |
| Siemens | S7-1500, S7-1200 | PROFINET, S7 Protocol | Supported |
| Schneider Electric | Modicon M580, M340 | Modbus TCP, EtherNet/IP | Supported |
| Beckhoff | TwinCAT 3 | ADS, OPC-UA native | Supported |
| Omron | NX/NJ Series | EtherNet/IP, EtherCAT | Supported |

### 3.1.2 Tag Naming Convention

A standardized tag naming convention is critical for abstraction. All PLC tags must follow this hierarchical naming pattern:

[Site]_[Area]_[Equipment]_[Module]_[Point]_[Attribute] Examples: BAL_MH1_MOLD01_HYD_PRESSURE_PV (Ballina, Molding Hall 1, Mold 01, Hydraulic, Pressure, Process Value) BAL_MH1_MOLD01_HYD_PRESSURE_SP (Setpoint) BAL_MH1_MOLD01_HYD_PRESSURE_HI (High Alarm Limit) BAL_MH1_MOLD01_HYD_PRESSURE_HIHI (High-High Alarm Limit) BAL_MH1_MOLD01_CYCLE_COUNT_PV (Cycle Counter Value) BAL_MH1_MOLD01_STATE_PV (Equipment State) BAL_MH1_MOLD01_FAULT_CODE (Active Fault Code)

### 3.1.3 Tag Naming Elements

| Element | Format | Description |
| --- | --- | --- |
| Site | 3 characters | BAL=Ballina, SDR=Stuarts Draft, LIB=Libertyville, KAU=Kaunas, FRE=Frederiksberg |
| Area | 2-4 characters | Production area code (MH1=Molding Hall 1, PKG=Packaging, CLN=Cleanroom) |
| Equipment | 6-10 characters | Equipment identifier matching asset registry (MOLD01, PRESS02, ROBOT03) |
| Module | 3-6 characters | Functional module within equipment (HYD=Hydraulic, CLAMP, INJECT, COOL) |
| Point | Variable | Measurement or control point (PRESSURE, TEMP, FLOW, SPEED, POSITION) |
| Attribute | 2-4 characters | PV=Process Value, SP=Setpoint, HI/LO=Alarm, CMD=Command, STS=Status |

### 3.1.4 Equipment State Model (PackML/ISA-88)

All equipment must implement a standardized state model based on PackML (ISA-TR88.00.02) for consistent status reporting:

| State ID | State Name | Description |
| --- | --- | --- |
| 0 | Undefined | State not determined or communication failure |
| 1 | Clearing | Fault is being cleared, machine preparing for restart |
| 2 | Stopped | Machine at rest, ready for commands |
| 3 | Starting | Machine ramping up to execute state |
| 4 | Idle | Machine running but not producing (waiting for material/command) |
| 5 | Suspended | Production paused due to upstream/downstream conditions |
| 6 | Execute | Machine actively producing |
| 7 | Stopping | Production stopping normally |
| 8 | Aborting | Emergency stop or fault condition stopping production |
| 9 | Aborted | Machine stopped due to abort condition, requires reset |
| 10 | Holding | Entering held state (operator pause) |
| 11 | Held | Production paused by operator, awaiting resume |
| 12 | Completing | Finishing current production cycle |
| 13 | Complete | Production batch/order complete |
| 14 | Resetting | Returning to idle state |

# 4. Edge Computing Layer (Layer 3)

## 4.1 Edge Gateway Architecture

The edge computing layer provides the critical abstraction between physical devices and enterprise systems. Edge gateways are deployed at each manufacturing facility to perform protocol translation, data normalization, and local processing.

### 4.1.1 Edge Gateway Functions

| Function | Description |
| --- | --- |
| Protocol Translation | Convert proprietary PLC protocols (EtherNet/IP, S7, Modbus) to standard protocols (OPC-UA, MQTT) |
| Data Normalization | Map vendor-specific data types to canonical formats, apply engineering unit conversions, standardize timestamps |
| Store and Forward | Buffer data locally when upstream connectivity is lost, replay when connection restored |
| Edge Analytics | Perform time-critical calculations locally (OEE, SPC, aggregations) to reduce latency |
| Data Filtering | Apply deadband filtering, downsampling, and exception-based reporting to reduce bandwidth |
| Security Gateway | Provide DMZ between OT and IT networks, enforce access policies, encrypt data in transit |

### 4.1.2 Recommended Edge Platforms

The following edge platforms are approved for deployment based on protocol support, scalability, and vendor neutrality:

| Platform | Strengths | Use Case |
| --- | --- | --- |
| Kepware KEPServerEX | 150+ device drivers, industry standard, proven reliability | Primary protocol translator for legacy and modern PLCs |
| Ignition Edge | MQTT-native, unlimited tags, open licensing model | High-volume data collection, store-and-forward scenarios |
| Azure IoT Edge | Container-based, AI/ML at edge, cloud integration | Edge analytics, predictive maintenance models |
| EdgeX Foundry | Open source (Linux Foundation), vendor neutral, microservices | Avoiding vendor lock-in, custom integration needs |

## 4.2 Protocol Standards

### 4.2.1 OPC-UA (Primary Standard)

OPC Unified Architecture is the primary protocol for PLC-to-edge communication. All new equipment must support OPC-UA natively or through gateway translation.

OPC-UA Configuration Requirements:

- Security Mode: SignAndEncrypt (minimum Sign for legacy)

- Security Policy: Basic256Sha256 (preferred) or Basic256

- User Authentication: Username/Password with certificate validation

- Session Timeout: 30 minutes (configurable)

- Publishing Interval: 100ms minimum for process data, 1s for status

### 4.2.2 MQTT (Event Messaging)

MQTT is used for lightweight, event-driven messaging from edge to enterprise, particularly for high-volume sensor data and real-time events.

MQTT Configuration Requirements:

- Protocol Version: MQTT 5.0 (preferred) or 3.1.1

- QoS Level: QoS 1 (at least once) for production data, QoS 2 for critical events

- TLS: Required for all connections (TLS 1.2 minimum)

- Authentication: Client certificates or username/password

- Topic Structure: hollister/{site}/{area}/{equipment}/{datatype}

MQTT Topic Hierarchy:

hollister/ ├── {site}/ # BAL, SDR, LIB, KAU, FRE │ ├── {area}/ # MH1, PKG, CLN │ │ ├── {equipment}/ # MOLD01, PRESS02 │ │ │ ├── state # Equipment state changes │ │ │ ├── production # Cycle complete, counts │ │ │ ├── process # Process measurements │ │ │ ├── quality # Inspection results │ │ │ ├── alarms # Active alarms │ │ │ └── maintenance # Maintenance events │ │ └── _area_summary # Aggregated area data │ └── _site_summary # Aggregated site data └── _enterprise # Global aggregations

# 5. Semantic and Context Layer (Layer 4)

## 5.1 Canonical Data Model

The semantic layer provides meaning to raw device data through canonical models based on ISA-95/IEC 62264 standards. This layer is the key to vendor independence—all integrations reference canonical models rather than physical device structures.

### 5.1.1 Equipment Hierarchy (ISA-95)

All equipment is organized according to the ISA-95 equipment hierarchy:

Enterprise (Hollister Incorporated) └── Site (Ballina, Stuarts Draft, Libertyville, Kaunas, Frederiksberg) └── Area (Molding Hall 1, Packaging, Cleanroom) └── Work Center (Catheter Line 01, Ostomy Line 02) └── Work Unit (MOLD-001, PRESS-002, ROBOT-003) └── Equipment Module (Hydraulic System, Cooling System) └── Control Module (Pressure Sensor, Temperature Probe)

### 5.1.2 Equipment Definition Schema

Each piece of equipment is defined in the canonical registry using this JSON schema:

{ "$schema": "https://hollister.com/schemas/equipment/v1", "equipmentId": "MOLD-001", "equipmentClass": "InjectionMoldingMachine", "description": "Arburg 520A Injection Molding Machine", "hierarchy": { "enterprise": "Hollister", "site": "BAL", "area": "MH1", "workCenter": "CATH-LINE-01", "workUnit": "MOLD-001" }, "location": { "building": "Building A", "floor": "Ground", "coordinates": { "x": 45.2, "y": 120.8 } }, "capabilities": [ { "capabilityType": "Molding", "capabilityValue": { "materials": ["HDPE", "LDPE", "PP", "TPE"], "maxShotSize": 250, "maxClampForce": 520, "maxMoldSize": { "width": 500, "height": 400 } } } ], "stateMachine": { "standard": "PackML", "stateTagPath": "opc.tcp://edge-bal-01:4840/ns=2;s=BAL_MH1_MOLD01_STATE_PV" }, "dataPoints": [ { "canonicalName": "CycleTime", "description": "Time for complete injection cycle", "dataType": "Float32", "engineeringUnit": "seconds", "physicalTag": "BAL_MH1_MOLD01_CYCLE_TIME_PV", "protocol": "OPC-UA", "endpoint": "opc.tcp://edge-bal-01:4840", "nodeId": "ns=2;s=BAL_MH1_MOLD01_CYCLE_TIME_PV", "sampleRate": "1s", "deadband": 0.1 }, { "canonicalName": "MeltTemperature", "description": "Barrel melt temperature", "dataType": "Float32", "engineeringUnit": "degC", "physicalTag": "BAL_MH1_MOLD01_BARREL_TEMP_PV", "protocol": "OPC-UA", "endpoint": "opc.tcp://edge-bal-01:4840", "nodeId": "ns=2;s=BAL_MH1_MOLD01_BARREL_TEMP_PV", "sampleRate": "5s", "limits": { "low": 180, "high": 280, "lowLow": 170, "highHigh": 290 } } ], "maintenanceSchedule": { "preventiveInterval": "500 hours", "lastService": "2026-01-15T08:00:00Z" }, "regulatory": { "validationStatus": "Validated", "lastValidation": "2025-09-01", "nextValidation": "2026-09-01" } }

## 5.2 Metadata Registry

The metadata registry is the central repository for all data definitions, providing the translation layer between physical tags and canonical names.

### 5.2.1 Registry Components

| Component | Description |
| --- | --- |
| Asset Registry | Complete inventory of equipment, sensors, and PLCs with hierarchy relationships |
| Tag Dictionary | Mapping between physical PLC tags and canonical data point names |
| Unit of Measure Catalog | Standard engineering units with conversion factors (SI preferred) |
| Data Type Registry | Canonical data types with validation rules and range limits |
| Equipment Class Library | Templates for equipment types with standard data points and capabilities |
| Protocol Endpoint Catalog | Connection details for all OPC-UA servers and MQTT brokers |

## 5.3 Data Governance

The governance engine enforces data quality, security, and compliance policies across all integration flows.

### 5.3.1 Governance Policies

- Data Lineage: Track origin of every data point from physical sensor through transformations to consumers

- Access Control: Role-based access to equipment data aligned with organizational structure

- Data Quality: Automated validation of values against defined ranges and patterns

- Change Control: All metadata changes require approval workflow with audit trail

- Retention Policies: Define storage duration based on regulatory requirements (FDA: 2 years beyond device lifetime)

# 6. Integration Platform (Layer 5)

## 6.1 Event Streaming Architecture

Apache Kafka serves as the central event streaming platform, providing durable, scalable, and decoupled communication between data producers and consumers.

### 6.1.1 Kafka Topic Structure

| Topic Pattern | Partitions | Purpose |
| --- | --- | --- |
| equipment.state.{site} | By equipment ID | Equipment state changes (PackML states) |
| production.events.{site} | By work order | Cycle complete, quantity produced, material consumed |
| process.measurements.{site} | By equipment ID | High-frequency process data (temp, pressure, speed) |
| quality.inspections.{site} | By work order | Inspection results, SPC data, pass/fail |
| alarms.active.{site} | By severity | Active alarms and acknowledgments |
| maintenance.events.{site} | By equipment ID | Maintenance requests, completions, PM due |
| commands.equipment.{site} | By equipment ID | Southbound commands to equipment |

### 6.1.2 Message Schema (Avro)

All messages conform to registered Avro schemas to ensure data contracts between producers and consumers:

{ "type": "record", "name": "ProductionEvent", "namespace": "com.hollister.manufacturing.events", "doc": "Event generated when production cycle completes", "fields": [ { "name": "eventId", "type": "string", "doc": "Unique identifier for this event (UUID)" }, { "name": "eventType", "type": { "type": "enum", "name": "ProductionEventType", "symbols": ["CYCLE_COMPLETE", "BATCH_START", "BATCH_END", "MATERIAL_CONSUMED", "SCRAP_PRODUCED"] } }, { "name": "timestamp", "type": "long", "logicalType": "timestamp-millis", "doc": "Event timestamp in UTC milliseconds" }, { "name": "equipmentId", "type": "string", "doc": "Canonical equipment identifier" }, { "name": "workOrderId", "type": ["null", "string"], "default": null, "doc": "Associated work order if applicable" }, { "name": "materialLot", "type": ["null", "string"], "default": null, "doc": "Material lot number consumed" }, { "name": "quantityGood", "type": ["null", "int"], "default": null }, { "name": "quantityReject", "type": ["null", "int"], "default": null }, { "name": "cycleTimeSeconds", "type": ["null", "float"], "default": null }, { "name": "processData", "type": { "type": "map", "values": "double" }, "doc": "Key-value pairs of process measurements", "default": {} }, { "name": "source", "type": { "type": "record", "name": "EventSource", "fields": [ {"name": "site", "type": "string"}, {"name": "edgeGateway", "type": "string"}, {"name": "originalTimestamp", "type": "long", "logicalType": "timestamp-millis"} ] } } ] }

## 6.2 API Gateway

The API gateway provides synchronous access to manufacturing data for applications that require request-response patterns.

### 6.2.1 API Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/v1/equipment/{id} | Get equipment definition and current state |
| GET | /api/v1/equipment/{id}/datapoints | Get current values of all data points |
| GET | /api/v1/equipment/{id}/history | Get historical data with time range |
| POST | /api/v1/equipment/{id}/commands | Send command to equipment (setpoint, recipe) |
| GET | /api/v1/production/orders/{id} | Get production order status and progress |
| GET | /api/v1/quality/inspections | Query inspection results with filters |
| GET | /api/v1/alarms/active | Get all active alarms across equipment |

# 7. MES Integration (Layer 6)

## 7.1 MES Adapter Pattern

The MES adapter translates between the canonical data model and MES-specific APIs. This pattern ensures that MES changes or replacements do not impact the underlying equipment integration.

### 7.1.1 Adapter Responsibilities

- Subscribe to relevant Kafka topics for equipment events

- Transform canonical events to MES-specific API calls

- Handle MES-specific authentication and session management

- Manage retry logic and error handling for MES communication

- Transform MES commands to canonical format for southbound flow

### 7.1.2 Critical Manufacturing MES Integration Points

| Integration | Direction | Data and Mapping |
| --- | --- | --- |
| Equipment State | PLC → MES | PackML state → CM Resource State (Available, Busy, Down, Maintenance) |
| Production Tracking | PLC → MES | Cycle complete event → CM TrackIn/TrackOut with quantity, lot, process data |
| Material Consumption | PLC → MES | Material consumed event → CM Material Tracking with lot genealogy |
| Quality Data | PLC → MES | Inspection result → CM Data Collection with parametric values |
| Work Order Download | MES → PLC | CM Work Order Release → Recipe parameters, setpoints, quantities |
| Label Data | MES → PLC | CM Lot/Serial generation → UDI, barcode data for label printing |

### 7.1.3 Adapter Implementation Pattern

The following pseudocode illustrates the adapter pattern for production event handling:

class MESEquipmentAdapter: """ Adapter translating between canonical events and Critical Manufacturing MES API. This is the ONLY component that knows MES-specific details. """ def __init__(self, registry: MetadataRegistry, mes_client: CMClient): self.registry = registry self.mes = mes_client self.kafka_consumer = KafkaConsumer(['production.events.*']) async def run(self): """Main event processing loop""" async for message in self.kafka_consumer: event = ProductionEvent.deserialize(message.value) await self.handle_production_event(event) async def handle_production_event(self, event: ProductionEvent): # 1. Lookup equipment in canonical registry equipment = self.registry.get_equipment(event.equipment_id) # 2. Map canonical equipment to MES resource mes_resource = self.get_mes_resource_mapping(equipment) # 3. Transform and send to MES based on event type if event.event_type == ProductionEventType.CYCLE_COMPLETE: await self.report_production_complete(event, equipment, mes_resource) elif event.event_type == ProductionEventType.MATERIAL_CONSUMED: await self.report_material_consumption(event, equipment, mes_resource) async def report_production_complete(self, event, equipment, mes_resource): # Transform canonical process data to MES parameters mes_data_collection = {} for canonical_name, value in event.process_data.items(): # Lookup MES parameter name from canonical mapping data_point = equipment.get_data_point(canonical_name) mes_param = self.registry.get_mes_parameter_mapping( data_point.canonical_name, mes_resource.resource_type ) if mes_param: # Apply any unit conversion if needed converted_value = self.convert_units( value, data_point.engineering_unit, mes_param.expected_unit ) mes_data_collection[mes_param.name] = converted_value # Call Critical Manufacturing API await self.mes.track_out( resource_id=mes_resource.id, work_order_id=event.work_order_id, quantity_good=event.quantity_good, quantity_reject=event.quantity_reject, cycle_time=event.cycle_time_seconds, data_collection=mes_data_collection, material_lot=event.material_lot ) def get_mes_resource_mapping(self, equipment: Equipment) -> MESResource: """ Map canonical equipment to MES resource. This mapping is stored in the metadata registry, not hardcoded. """ return self.registry.get_mes_mapping( equipment.equipment_id, target_system="CriticalManufacturing" )

# 8. Security Architecture

## 8.1 OT/IT Network Segmentation

The architecture implements defense-in-depth with clear separation between Operational Technology (OT) and Information Technology (IT) networks.

### 8.1.1 Network Zones

| Zone | Components | Security Controls |
| --- | --- | --- |
| Level 0-1 | PLCs, Sensors, Actuators | Air-gapped from IT, physical security, no direct internet |
| Level 2 | SCADA, HMI, Historians | OT network, firewall protected, limited IT access |
| Level 3 (DMZ) | Edge Gateways, Protocol Translators | Industrial DMZ, dual firewalls, data diode where required |
| Level 4 | MES, Integration Platform | IT network with OT access policies, encrypted communication |
| Level 5 | ERP, Analytics, Cloud | Enterprise IT security controls, internet access |

## 8.2 Authentication and Authorization

- OPC-UA: X.509 certificates for server/client authentication, username/password for user auth

- MQTT: TLS client certificates, OAuth 2.0 tokens for service accounts

- Kafka: SASL/SCRAM authentication, ACLs for topic-level authorization

- APIs: OAuth 2.0 / OpenID Connect, JWT tokens with short expiration

- MES: Integration with enterprise identity provider (Azure AD/Okta)

## 8.3 Regulatory Compliance (21 CFR Part 11)

All components handling electronic records must comply with FDA 21 CFR Part 11:

- Audit Trails: Every data transaction logged with timestamp, user, before/after values

- Electronic Signatures: Biometric or two-factor authentication for approvals

- Access Controls: Role-based permissions aligned with job responsibilities

- Record Integrity: Cryptographic hashing to detect unauthorized modifications

- System Validation: IQ/OQ/PQ protocols for all GxP-critical systems

# 9. Implementation Guide

## 9.1 Equipment Onboarding Checklist

Follow this checklist when integrating new equipment into the Digital Factory:

| Step | Task | Deliverable |
| --- | --- | --- |
| 1 | Document equipment capabilities and I/O points | Equipment specification sheet |
| 2 | Assign equipment ID and hierarchy location | Asset registry entry |
| 3 | Define tag naming following conventions | Tag list document |
| 4 | Configure PLC with standard state model | PLC program with PackML states |
| 5 | Configure edge gateway connection | OPC-UA/MQTT connectivity verified |
| 6 | Create canonical equipment definition | JSON definition in metadata registry |
| 7 | Map data points to MES parameters | Mapping configuration |
| 8 | Configure Kafka topics and subscriptions | Topic configuration, ACLs |
| 9 | Test end-to-end data flow | Integration test results |
| 10 | Execute validation protocol (if GxP) | IQ/OQ/PQ documentation |
| 11 | Go-live and monitor | Operational handover |

## 9.2 Troubleshooting Guide

| Symptom | Possible Cause | Resolution |
| --- | --- | --- |
| No data from PLC | OPC-UA connection failure | Check certificates, firewall rules, PLC OPC-UA server status |
| Data appears in Kafka but not MES | MES adapter not processing | Check adapter logs, verify schema compatibility, MES API connectivity |
| Incorrect values in MES | Unit conversion or mapping error | Verify canonical model, check unit conversion configuration |
| High latency in data | Network congestion or polling interval | Review edge gateway sampling rates, network bandwidth |
| Missing events after network outage | Store-and-forward not enabled | Configure edge gateway buffering, verify replay on reconnect |

# Appendix A: OPC-UA Information Models

The following OPC-UA companion specifications should be implemented where applicable:

| Specification | OPC Number | Application |
| --- | --- | --- |
| OPC-UA for Machinery | OPC 40001-1 | Base model for all production machinery |
| OPC-UA for PackML | OPC 40200 | Packaging machinery state model |
| OPC-UA for ISA-95 | OPC 40600 | MES integration data model |
| OPC-UA for Robotics | OPC 40010 | Industrial robot integration |
| OPC-UA for Machine Vision | OPC 40100 | Vision inspection systems |

# Appendix B: Reference Architecture Diagram

The following diagram illustrates the complete PLC-to-MES integration architecture:

┌─────────────────────────────────────────────────────────────────────────────┐ │ APPLICATION LAYER (Level 6) │ │ ┌─────────────┐ ┌─────────────┐ ┌─────────────┐ ┌─────────────┐ ┌──────────┐ │ │ │ Critical │ │ Digital │ │ AI/ML │ │ Historian │ │ SAP ERP │ │ │ │Manufacturing│ │ Twin │ │ Analytics │ │ (OSIsoft) │ │ S/4HANA │ │ │ │ MES │ │ Platform │ │ Platform │ │ │ │ │ │ │ └──────┬──────┘ └──────┬──────┘ └──────┬──────┘ └──────┬──────┘ └────┬─────┘ │ └─────────────┼─────────────┼─────────────┼─────────────┼─────────────┼─────────┘ │ │ │ │ │ ┌─────────────┴─────────────┴─────────────┴─────────────┴───────────────────────┐ │ INTEGRATION PLATFORM (Level 5) │ │ ┌───────────────────────────────┐ ┌──────────────────────────────┐ │ │ │ Apache Kafka Cluster │ │ API Gateway │ │ │ │ (Event Streaming + Schema Reg) │ │ (REST/GraphQL + Auth) │ │ │ └───────────────┬───────────────┘ └──────────────┬───────────────┘ │ └─────────────────┼──────────────────────────────┼───────────────────────────┘ │ │ ┌─────────────────┴──────────────────────────────┴───────────────────────────┐ │ SEMANTIC LAYER (Level 4) │ │ ┌────────────────────┐ ┌───────────────────┐ ┌────────────────────┐ │ │ │ Canonical Data │ │ Metadata Registry │ │ Governance Engine │ │ │ │ Model (ISA-95) │ │ (Asset + Tag Dict)│ │(Lineage + Security)│ │ │ └────────────────────┘ └───────────────────┘ └────────────────────┘ │ └─────────────────────────────────────────────────────────────────────────────┘ │ ┌─────────────────────────────────────┴─────────────────────────────────────┐ │ EDGE COMPUTING LAYER (Level 3) │ │ ┌───────────────┐ ┌───────────────┐ ┌───────────────┐ ┌──────────────┐ │ │ │ OPC-UA Server │ │ MQTT Broker │ │ Edge Analytics│ │Store+Forward│ │ │ │ (Kepware) │ │ (HiveMQ) │ │ (Local Calc) │ │ (Buffer) │ │ │ └───────┬───────┘ └───────┬───────┘ └───────┬───────┘ └──────┬───────┘ │ │ └─────────────┼───────────────┼─────────────┘ │ │ │ │ │ ┌───────────────────────────┴────────────────────────────────────────┐ │ │ │ PROTOCOL TRANSLATOR / ADAPTER LAYER │ │ │ │ ┌───────────┐ ┌───────────┐ ┌───────────┐ ┌─────────────┐ │ │ │ │ │EtherNet/IP│ │ PROFINET │ │ Modbus TCP│ │ S7 Protocol│ │ │ │ │ │ Driver │ │ Driver │ │ Driver │ │ Driver │ │ │ │ │ └─────┬─────┘ └─────┬─────┘ └─────┬─────┘ └──────┬──────┘ │ │ │ └───────────┼───────────┼───────────┼───────────┼──────────────────┘ │ └─────────────┼───────────┼───────────┼───────────┼──────────────────────────┘ │ │ │ │ ┌─────────────┴───────────┴───────────┴─────────────────────────────────────┐ │ PHYSICAL DEVICE LAYER (Level 1-2) │ │ ┌───────────┐ ┌───────────┐ ┌───────────┐ ┌───────────┐ ┌────────┐ │ │ │Allen-Brad.│ │ Siemens │ │ Generic │ │ Vision │ │ Legacy │ │ │ │ControlLogix│ │ S7-1500 │ │ Modbus │ │ Systems │ │ Serial │ │ │ └───────────┘ └───────────┘ └───────────┘ └───────────┘ └────────┘ │ └─────────────────────────────────────────────────────────────────────────────┘

End of Technical Specification